7-1 Project

Samuel Roulstone

Southern New Hampshire University

IT-212-17363-M01 Intro to Computer Networks

Professor Jim Crivello

June 22, 2025

As the advertising firm expands its presence beyond the Northeast, the establishment of a new office in Fayetteville, North Carolina, marks a significant milestone in its strategic growth under the Future Vision initiative. With the Albany headquarters serving as the corporate backbone, housing the executive team and core IT services, and the Springfield office supporting a highly mobile workforce, the new Fayetteville office must be designed to seamlessly integrate with the existing infrastructure while maintaining performance, security, and scalability. The network must support 50 employees, including a newly appointed executive vice president, and ensure that the systems function in real time with minimal latency across all sites. This includes supporting site-to-site VPN connectivity, remote printing to billboard printers in Albany, and high-definition video conferencing with other locations.

Building the Fayetteville network requires careful consideration of both technical and strategic needs. The physical infrastructure will be rooted in a fiber-optic internet connection, as this offers the highest reliability, bandwidth, and lowest latency among the available options in the area. The fiber-optic solution is ideal for transmitting large media files and conducting smooth video conferencing sessions, which are critical for internal collaboration and client-facing activities. The internal network will be supported by Cat6 structured cabling capable of 10 Gbps speeds, complemented by Wi-Fi 6 access points to ensure reliable wireless coverage throughout the office. These components reflect the foundational layer of the OSI model and establish a physical and data-link framework that supports robust network performance.

A central tenet of the network design is its alignment with the OSI model, which underpins modern communication networks. Each layer—from physical transmission to application-level protocols—has been considered to ensure seamless interaction between systems. For example, managed Layer 3 switches will handle traffic routing and inter-VLAN communication (network layer), while secure VPN connections ensure session integrity (session layer). The design also supports robust application-layer services such as video conferencing and cloud-based printing. By structuring the network around these principles, the proposed setup ensures clarity in configuration, ease of troubleshooting, and compatibility with enterprise-grade tools and standards.

IP addressing and scalability are also key considerations. A private Class C IP range of 192.168.3.0/24 is recommended, providing 254 usable addresses—more than sufficient for current and projected growth. Static IPs will be assigned to critical infrastructure such as servers, printers, and access points, while dynamic addressing through DHCP will be used for employee workstations and mobile devices. To enhance performance and security, the network will be logically segmented using VLANs. These will separate traffic based on function or department—for example, marketing, finance, guest Wi-Fi, and VoIP—ensuring better bandwidth allocation, improved security, and easier traffic management.

For the local area network (LAN) topology, a star configuration has been identified as the optimal choice. This structure connects all devices to a central switch, allowing for straightforward management, simplified troubleshooting, and fault isolation. Should any individual workstation or connection fail, the rest of the network remains unaffected. While the central switch represents a potential point of failure, this risk is mitigated by deploying enterprise-grade equipment with redundant power supplies. Compared to more complex or outdated topologies—such as mesh, ring, or bus—the star topology offers the right balance of performance, simplicity, and cost-effectiveness for a mid-sized office like Fayetteville.

Internet connectivity is fundamental to the success of the Fayetteville office, given the reliance on cloud services, VPNs, and digital collaboration tools. After evaluating local ISPs, AT&T Fiber is the most appropriate option due to its symmetric upload/download speeds of up to 5 Gbps, low latency, and business-grade service-level agreements. In contrast, other providers like Spectrum Business offer less consistent upload speeds and higher latency, which could interfere with critical business functions. By selecting AT&T Fiber, the Fayetteville office will meet and exceed performance expectations, support business continuity, and ensure high-quality communications with Albany and Springfield.

The hardware setup will include high-performance components necessary to maintain security and performance. A next-generation firewall will protect against external threats and support VPN tunnels with encryption and intrusion prevention systems. Layer 3 managed switches with Power over Ethernet will provide flexibility in powering VoIP phones and access points. High-performance wireless access points based on Wi-Fi 6 will support dense device usage. To ensure reliability and minimize disruptions, an uninterruptible power supply (UPS) will be used for key components, and patch panels will keep structured cabling organized and scalable. On the software side, network monitoring tools such as SolarWinds or PRTG will allow remote IT administrators from Albany to monitor performance and respond to issues in real time. All devices will include endpoint protection software managed by the centralized IT team, ensuring consistent security policies and configurations across the entire organization.

The Fayetteville office will also require an effective remote printing solution to interface with printers located in Albany. For this, cloud-based print management software like Microsoft Universal Print or PaperCut MF will be deployed. These platforms enable secure transmission of print jobs over the network, automate driver distribution to new devices, and enforce print access controls. This not only reduces administrative overhead but also ensures secure and reliable document delivery across office locations.

Video conferencing is a core requirement for the Fayetteville site, especially given the presence of senior leadership. High-quality video conferencing depends on bandwidth and quality of service (QoS) configurations. At least 100 Mbps of dedicated bandwidth will be allocated for video traffic, and routers will prioritize video and voice packets to maintain stability and avoid jitter. Conference rooms will be equipped with 4K displays, HD cameras, beamforming microphones, and high-quality speakers. Integrated conferencing platforms like Zoom Rooms or Microsoft Teams Rooms will ensure consistent user experiences and compatibility across all offices.

Anticipating potential network errors is critical to ensuring continuity. Common issues might include VPN tunnel instability, hardware failures, bandwidth saturation, and wireless interference. To address these, a robust troubleshooting process will be established. Tools like ping and traceroute will help isolate problems, while interface monitoring and syslog analysis will offer deeper insights into network performance. By proactively monitoring network health, the Albany-based IT team can quickly identify and resolve issues—even without being physically present. In addition, hardware redundancy and failover configurations will further minimize downtime and disruption.

To ensure the network remains efficient and secure over time, industry-standard practices must be implemented for monitoring and maintenance. Port scanning and packet flow analysis will be used to identify anomalies and potential intrusions. SNMP-based monitoring will provide real-time visibility into switch and router performance. Automated patch management tools such as WSUS or PDQ Deploy will help the IT department quickly test, deploy, and verify updates across all systems. For inventory management, tools like Lansweeper or ManageEngine will track device locations, software versions, and licensing status. This comprehensive approach ensures the infrastructure remains secure, compliant, and ready for future expansion.

In conclusion, the network infrastructure for the Fayetteville office must reflect the same performance, security, and operational standards found in the Albany and Springfield offices. By deploying a fiber-based internet connection, implementing a reliable star topology, and using enterprise-grade hardware and software, the Fayetteville office will be well-equipped to meet current business demands and scale with future growth. This proposal supports the company’s strategic goals under the Future Vision initiative and establishes a robust foundation for continued expansion and innovation in a digitally connected environment.

References

AT&T. (2024). Business fiber internet plans. AT&T Business.

https://www.business.att.com/products/business-fiber.html

Cisco. (2023). Small business networking: Build a foundation for growth. Cisco Systems, Inc.

https://www.cisco.com/c/en/us/solutions/small-business/networking.html

Fortinet. (2023). What is a firewall and why is it important? Fortinet, Inc.

https://www.fortinet.com/resources/cyberglossary/firewall